Mouse Arena Ethanol Console

User Guide — setting per-trial odor-contact thresholds

Version 0.1.0  ·  Mouse Arena repository  ·  runs in the ma-console environment

The Ethanol Console is a focused companion to the Data Console. Where the Data Console browses whole selections of trials, the Ethanol Console works one trial at a time so you can look closely at each ethanol trace, set an accurate odor-contact threshold, and save it for downstream analyses. It reads the same aggregate through the same accessor and uses the same arena frame and file-name conventions.

# 1  What the console shows

For the currently selected trial, three linked panels are drawn:

- Trajectory map (left) — the head track in the arena pixel frame (0–580 × 0–280, y downward), a red dot at every odor-contact onset, and a red circle at the target odor port (the reward endpoint).

- Binary preview (top right) — a green band showing where odor is ON versus OFF along time at the current threshold (qualifying contact episodes only).

- Thresholding trace (bottom right) — the baseline-subtracted ethanol signal with the draggable red threshold line; each qualifying episode is shaded. The preview shares this panel's time axis, so zooming the trace zooms the preview too.

The number of contacts and their Euclidean distance from the odor port (min / mean / max, in pixels) update live in the control bar as you move the threshold.

# 2  Environment and launch

The console runs in the same conda-forge environment as the Data Console (ma-console). No SciPy is required — the baseline is pure NumPy. If you have not created the environment yet:

conda create -n ma-console -c conda-forge python=3.12 pyside6 matplotlib numpy h5py

Then launch with either:

run_ethanol_console.bat  ["path\to\Mouse Arena Aggregate Data.h5"]

python run_ethanol_console.py  ["path\to\...h5"]

With no path argument it uses the default repository location, ..\..\DATA\Mouse Arena Aggregate Data.h5. The .bat finds the ma-console Python automatically (falling back to conda run).

# 3  The interface

All controls sit in the bar above the panels. The nav toolbar (small icons at the very top of the plot area) provides box-zoom, pan, and home/back/forward.

| Control | What it does |
| --- | --- |
| IR status / Date / End loc | Narrow the set of trials you step through (same selectors as the Data Console). |
| Trial ▾ · ◀ Prev · Next ▶ · ← / → arrows | Move through the filtered trials one at a time. A ✓ marks trials already saved. |
| Signal: Deconvolved / Raw | Which trace to threshold: ethdeconv (default) or raw ethanol. |
| Baseline subtract · pct · win(s) | Subtract a rolling low-percentile baseline before thresholding (default 10th percentile, 20 s window). |
| Min dur (s) | Minimum above-threshold episode duration to count as a contact (default 0.05 s). |
| Threshold spin · drag red line · Auto | Set the threshold in baseline-subtracted signal units. Auto = median + k·MAD. |
| Save (Enter) | Write this trial's threshold and contacts to the master JSON (and the DATA mirror). |
| Box-zoom / pan toolbar | Click the zoom tool, then drag a box on the trace to zoom both axes; Home resets. |

# 4  Thresholding workflow

- Pick the trial with the selectors, the Trial dropdown, or the ← / → arrow keys.

- Confirm the signal (Deconvolved by default) and that Baseline subtract is on.

- Set the threshold — either drag the dashed red line up/down on the trace, type a value in the Threshold box, or click Auto for a median + k·MAD starting point.

- Watch the binary preview and the contact count. Zoom in (box-zoom) on ambiguous stretches to judge real odor encounters versus noise; adjust Min dur (s) to reject brief spikes.

- Press Enter (or click Save) to persist the threshold. The trial gets a ✓ and the status bar confirms the file paths.

- Advance to the next trial. When you return to a saved trial, its threshold is restored automatically.

Seeding: on landing on a trial the threshold seeds from the saved JSON value if that trial was thresholded before with the same signal; otherwise it seeds from the Auto estimate.

# 5  How contacts are computed

## Baseline subtraction

A rolling low-percentile baseline (default: 10th percentile in a 20 s window) is subtracted from the chosen trace before thresholding. This removes slow drift so the threshold sits on top of the local noise floor rather than an absolute level. Turn it off, or change the percentile / window, in row 2.

## Contacts = min-duration crossings

An odor contact is a contiguous run of samples at or above the threshold whose duration is at least Min dur (s). Runs shorter than that are ignored, which suppresses single-sample noise spikes. Each contact's onset is the first sample of the run.

## Shared-origin clock alignment

The head track and the ethanol trace share a seconds origin, but the head track can begin several seconds after t = 0. The console aligns on that shared origin (it does not zero-base each stream independently), so a contact's on-trace time maps to the correct head position. Contacts that fall before head tracking begins have no position and are omitted from the map and the distance list.

## Distance to source

Each contact's head position at onset is compared to the trial's odor port (the endpoint) by straight-line pixel distance. The control bar summarizes these as min / mean / max; the full per-contact list is saved to the JSON.

# 6  Saving and the JSON schema

Saving writes one master file, ethanol_thresholds.json, keyed by recording (file) name, to two places:

- Ethanol Console\output\ethanol_thresholds.json  (primary)

- DATA\ethanol_thresholds.json  (mirror, next to the aggregate)

The file carries production timestamps and one entry per thresholded trial. File-level metadata:

| meta field | Meaning |
| --- | --- |
| schema | Schema id (mouse_arena/ethanol_thresholds/1.0). |
| created_utc | When the file was first created (UTC). |
| updated_utc | When the file was last written (UTC). |
| generated_local | Last write in local time, with timezone. |
| aggregate_path | The aggregate the thresholds were set against. |

Each trial entry (under "trials", keyed by file_name):

| trial field | Meaning |
| --- | --- |
| trial_index | 0-based trial index in /behavior. |
| loc / animal / date / lighting | Parsed from the file name (odor-port Loc, animal id, date, IR status). |
| signal | deconv or raw — which trace was thresholded. |
| baseline_subtracted / baseline | Whether baseline subtraction was on, and its method/pct/window/step. |
| threshold | The threshold value, in the (baseline-subtracted) signal units. |
| threshold_units | e.g. ethdeconv_baseline_subtracted — how to interpret the threshold. |
| min_duration_s | Minimum episode duration used for contacts. |
| n_contacts | Number of qualifying contacts. |
| contact_onsets_s | Onset time (s, shared origin) of each contact. |
| contact_distances_px | Euclidean px distance of each contact from the odor port (null if position unavailable). |
| endpoint_px | The odor-port location [x, y]. |
| set_utc / set_local | When this trial's threshold was saved. |

# 7  Using the thresholds downstream

To reproduce a trial's odor-on mask in an analysis: take the named signal, baseline-subtract it with the recorded baseline params, then keep above-threshold episodes of at least min_duration_s. The console's own detection functions are importable and do exactly this:

import sys; sys.path.insert(0, r"...\Ethanol Console\code")

from ethanol_console.data import rolling_percentile_baseline, detect_contacts

base = rolling_percentile_baseline(sig, fs, win_s=20, pct=10)

contacts = detect_contacts(t, sig - base, thr, min_duration_s=0.05)

Because the JSON stores every parameter used, an analysis never has to guess how a threshold was produced — read the entry, apply the same steps, and the contacts match what you saw on screen.

# 8  Tips and troubleshooting

- Arrow keys not advancing the trial? Click once on the plot area so the window has keyboard focus, then use ← / →. The Prev/Next buttons always work.

- The red threshold line won't drag while a zoom or pan tool is active — click the toolbar tool off (or press Home) first, then drag.

- Too many contacts that look like noise: raise the threshold, increase Min dur (s), or confirm baseline subtraction is on.

- Contacts missing from the map: they occurred before head tracking began (outside head coverage); they are still counted on the trace but have no position.

- Raw vs deconvolved thresholds are not interchangeable — they live on different scales. The JSON records signal and threshold_units so downstream code applies each correctly.

- Qt DLL errors on launch: use the ma-console environment (not the vras/Anaconda base Python), exactly as for the Data Console.